# Shieldmark Rack Protector Design Specification

Revision History

| 0.1 | Initial draft |
| --- | --- |

Table of Contents

1.	Preface	2

2.	Functionality Description	2

3.	Requirements	2

4.	Design Overview	2

5.	Hardware Design:	3

6.	Firmware Design	4

7.	References	5

8.	Operational Flowchart	6

## Preface

Section to describe abbreviations or define unique terms.

## Functionality Description

The Rack Protector is a microcontroller-based lighting and alert system designed to detect the presence of Tow-Motors in warehouse environments to help prevent collisions with storage racks and shelving. The system serves as a safety device that continuously monitors for approaching Tow-Motors and provides both visual and audible feedback via bright-white LEDs and audible sound to alert operators and nearby personnel.

When powered on, the Rack Protector initializes its microcontroller and begins monitoring proximity data received from a dedicated infrared (IR) sensor module. This sensor board communicates with the main controller module over an I²C interface, transmitting real-time presence data that the firmware interprets to determine how close a Tow-Motor is to the area. When the microcontroller detects presence within a programmed threshold, it responds by illuminating high-intensity LED arrays and by triggering a buzzer to provide a distinct audible warning.

Lastly, the rack protector provides battery level detection, which blinks both channels of LEDs

## Requirements

| REQ1 | Support Sensing from IR Sensor Board |
| --- | --- |
| REQ2 | Support 2 channels of 8 LEDs |
| REQ3 | Support Audio from Magnetic Buzzer |
| REQ4 | Support Power Management |
| REQ5 | Support Battery Dependent Power |
| REQ6 | Support Serial Data Connection |
| REQ7 | Support Battery Voltage Detection |

## Design Overview

The Rack Protector design consists of, in total, roughly:

- 1 × PIC18F26-45-46-Q10 Microcontroller

- 1 × Buck-Boost Converter IC

- 1 × 3.3 V Low Dropout Regulator

- 6–8 × MOSFETs/Transistors

- 16 × High-brightness LEDs

- 1 × Buzzer

- ~40 × Passive components (resistors, capacitors, inductors)

- 6 × Connectors/Headers

## Hardware Design:

Below is a table of each on board connection header along with it’s corresponding MCU Pinout

| Connector Header | Description | MCU Pins |
| --- | --- | --- |
| J1 | Male Headers for PICKIT Programmer | RB6/KBI2/PGC RB7/KBI3/PGD |
| J2 | Male Headers for Serial Connection | RC6/TX/CK RC7/RX/DT |
| J3, J5 | Connectors for Battery Power | RB3/CANRX RB4/KBIO/AN9 |
| J6 | Connector for IR Sensor | RC2/CCP1 RC3/SCK/SCL RC4/SDI/SDA |

5.1: Power:

The Rack Protector is powered by six C-sized alkaline batteries, arranged in two groups connected through separate connectors that are then wired in series on the PCB to produce a nominal 9 V DC supply.

5.2: LEDs:

The board drives 16 high-brightness white LEDs arranged into two independent LED banks. Each bank consists of 8 LEDs with current-limiting resistors and is switched by an N-channel MOSFET under control of the MCU. The control signals are labeled WHITE_EN and WHITE_EN2, corresponding to LED Bank 1 and LED Bank 2 respectively. When the microcontroller detects presence from the sensor, it sets these output pins high, turning on the MOSFETs and activating the LED banks. The 5 V rail provides LED power, while the microcontroller logic controls the timing and duty cycle for visual alerts or illumination.

5.3: Sensor Board:

The sensor board connects to the main PCB through a 5-pin I²C header (J6), carrying SDA, SCL, INT, VCC (3.3 V), and GND. The external infrared presence sensor continuously measures proximity and transmits digital readings over the I²C bus. The sensor board provides presence data that the MCU uses to determine when to trigger LEDs or the buzzer.

5.4: Serial Port:

A UART serial interface is provided at connector J2, allowing communication with a PC via USB-to-TTL adapter for debugging or monitoring through PuTTY.
The serial configuration is 115200 baud, 8 data bits, no parity, 1 stop bit (8-N-1) 
The transmit and receive lines use the MCU’s RC6 (TX1) and RC7 (RX1) pins, which are routed through the header.

5.5: Buzzer:

An audio buzzer (SPK1) provides an audible warning single-tone sound when motion or proximity is detected.

5.6: Battery Level Circuit:

The battery level monitoring circuit allows the microcontroller to measure the remaining battery voltage. The circuit is enabled by an input that is set high from the MCU labelled ANA_V_EN. A resistor voltage divider then scales the 9 V battery down to a safe range for the MCU’s analog-to-digital converter (ADC) input, labeled ANA_V_READ. By sampling this voltage, the firmware can calculate the actual battery level and compare it against predefined thresholds to decide when the rack protector should enter a low-power state.

## Firmware Design

There are two basic operational modes to be aware of within the firmware of the Rack Protector board, Normal and Low Power modes. Normal Mode represents the standard operating state of the Rack Protector, in which the firmware executes a continuous main loop that monitors proximity data and drives the visual and audible indicators accordingly. Presence readings are obtained from the infrared sensor board over the I²C interface at approximately 500 ms intervals, and the firmware classifies these readings into three response levels: Idle (LEDs off), Near (LEDs solid on), and Strong (LEDs blinking). During Strong-range detection, the buzzer output is also enabled to provide an additional audible warning. In this mode, all sensing, LED control, and serial reporting functions run normally and without restriction.

Low Power Mode is entered automatically from within the main loop when the measured battery voltage drops below a defined threshold of approximately 7.25 V. In this mode, the Rack Protector continues to operate normally—processing presence readings, updating LED indicators, and reporting data over UART—but adds a periodic notification behavior to warn the user of the low battery condition. Every ~30 seconds, the firmware performs a rapid-blink sequence on both LED banks, flashing each LED channel several times in succession. No hardware shutdown occurs; instead, the system provides a visual reminder that battery replacement maintenance is required while still performing independent of the standard sensing and alert functions.

During both operational modes, the battery voltage level along with the IR sensor values can be read over the serial port headers. Additionally, the firmware version is read out at board startup.

Firmware updates:

Requires re-flash through programming header J1 via PICKIT 5 programmer (no bootloader implemented)

Firmware Configuration:

Currently using MPLAB IDE ver. 6.25 with XC8 ver. 3.10 compiler to program/develop firmware

Operational modes:

- Normal; behavior is fully automatic during startup

- Low Power, normal behavior is still fully operational during this mode

Typical operation:
Power-on → Sensor initialized → Loop: Read Presence/Motion → Read Battery Level → Send battery value over UART → Apply threshold logic → If battery < 7.25 V enter Low Power behavior → Update LEDs/Buzzer based on presence → Perform periodic low-battery LED blink (~30 s interval) if in Low Power Mode → Repeat every ~500 ms

Startup behavior:

500 ms delay allows sensor boot time before register configuration
LEDs begin in OFF state, buzzer forced OFF at start.

Presence thresholds determine LED behavior:

Near range: ~0x00C0–0x0500 → LEDs ON solid

Strong range: >0x0500 & <0x7d00 → LEDs blink (toggle)

Out-of-range: values are capped/zeroed to prevent false states

- Uses a write command that reads back to the user what firmware is installed onto the board

## References

“PIC18F26-45-46-Q10-Data-Sheet-DS40001996.pdf”

“Rack_Protector Rev B Schematic.pdf”

“MPLAB-PICkit-5-In-Circuit-Debugger-Users-Guide-50003525.pdf”

https://onlinedocs.microchip.com/oxy/GUID-CE8CBDF8-D51B-4BC5-8EF8-F01B7427113D-en-US-4/GUID-F388E419-F13C-469D-8806-4DCC80E8192D.html

## Operational Flowchart